4. GAMMA RAY SPECTROMETRY (GRS)

# Silviu Olariu, Agata Olariu

###### Department of Nuclear Physics,

National Institute for Physics and Nuclear Engineering- Horia Hulubei, Magurele

4.1. Introduction

The project GRS is part of the project of package of diagnostic enhancements to be implemented during the Framework Programme 7. The objective for the year 2007 was the installation and the availability of intense gamma ray and neutron sources at the tandem accelerator of  IFIN-HH, Magurele-Bucharest,  in order to carry out testing of three new spectrometers, with complementary performance: a high energy resolution HPGe spectrometer and two high efficiency, high rate spectrometers with large detection crystal. The National Institute of Physics and Nuclear Engineering (IFIN-HH)  participates in this project on workpackage WP4, entitled Detector tests in high neutron/gamma flux conditions, in collaboration with Istituto di Fisica del Plasma (IFP),  Milan, Italy, via Consiglio Nazionale delle Ricerche, EURATOM - ENEA - CNR Association.

- 4.2. Results obtained in 2008

4.2.1 Generation of gamma-ray photons in the 15-25 MeV energy range for nuclear processes taking place in fusion plasmas

4.2.2 Nuclear capture reactions with Q>15000 keV

The generation of gamma-ray photons in the 15-25 MeV energy range has been studied for nuclear processes taking place in fusion plasmas. The nuclear capture reactions (p,g), (d, g), (t, g), (3He, g), (a, g) have been considered for stable low-mass elements extant in the fusion plasma, and those reactions for which the gamma-ray energy is larger than 15 MeV has been determined as follows:

| Reactia (p,) | Reactia (d,) | Reactia (3He,) |
| --- | --- | --- |
| Target; Q, keV; Product. 3H; 19813.9; 4He+. 7Li; 17255.44; 8Be+. 11B; 15956.9; 12C+. | Target; Q, keV; Product. 2H; 23846.53; 4He+. 3H; 16699.30; 5He+. 3He; 16388.00; 5Li+. 6Li; 22280.84; 8Be+. 7Li; 16696.22; 9Be+. 9Be; 15815.44; 11B+. 10B; 25186.46; 12C+. 11B; 18678.64; 13C+. 13C; 16159.30; 15N+. 14N; 20736.14; 16O+. 19F; 17380.11; 21Ne+. 21Ne; 16933.80; 23Na+. 22Ne; 16282.91; 25Na+. | Target; Q, keV; Product. 6Li; 16602.324; 9B+. 7Li; 17788.625; 10B+. 9Be; 26278.865; 12C+. 10B; 21636.465; 13N+. 11B; 20735.725; 14N+. 13C; 22793.225; 16O+. 14C; 18759.916; 17O+. 14N; 15842.926; 17F+. 17O; 21164.336; 20Ne+. 18O; 19881.465; 21Ne+. 19F; 18626.266; 22Na+. 21Ne; 23133.006; 24Mg+. 22Ne; 20099.326; 25Mg+. |

4.2.3 High-energy gamma rays produced in the nuclear capture reactions

For the aforementioned reactions, the nuclear levels have been determined for which the gamma-ray energy can be larger than 10 MeV. The gamma-ray lines identified in this way are situated in a spectral region where the contribution to the gamma-ray spectrum arising from the reactions due to the neutrons is relatively low, although at the same the efficacity of the detectors is also smaller.

| Nucleus | Elevel(keV) | Jπ | T1/2 | Eγ(keV) | Iγ | γ mult. | γ mix. ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9BE | 14392.2 18 | 3/2- | 365 eV 29 | 11954.3 22 | 100 5 | M1 |  |
| 9BE | 14392.2 18 | 3/2- | 365 eV 29 | 14380.0 18 | 88.3 44 | M1 |  |
| 9BE | 16977.1 5 | 1/2- | 389 eV 10 | 14.19E3 12 | 13.3 42 | M1 |  |
| 9BE | 16977.1 5 | 1/2- | 389 eV 10 | 14535.1 14 | 3.3 7 | E2 |  |
| 9BE | 16977.1 5 | 1/2- | 389 eV 10 | 16959.9 5 | 100 6 | M1 |  |

| Nucleus | Elevel(keV) | Jπ | T1/2 | Eγ(keV) | Iγ | γ mult. | γ mix. ratio | γ conv. coeff. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12C | 15110 3 | 1+ | 43.6 eV 13 | 10666.00 |  | M1 |  |  |
| 12C | 16105.8 7 | 2+ | 5.3 keV 2 | 16094.2 |  | E2 |  |  |

| Nucleus | Elevel(keV) | Jπ | T1/2 | Eγ(keV) | Iγ | γ mult. | γ mix. ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 13C | 15108.2 12 | 3/2- | 5.49 keV 25 | 11418.3 12 | 79 11 | [M1] |  |
| 13C | 15108.2 12 | 3/2- | 5.49 keV 25 | 12012.8 12 | 18 3 | [E1] |  |
| 13C | 15108.2 12 | 3/2- | 5.49 keV 25 | 15098.8 12 | 100 7 | [M1+E2] | 0.164 15 |

| Nucleus | Elevel(keV) | Jπ | T1/2 | Eγ(keV) | Iγ | γ mult. | γ mix. ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 13N | 1350E+1 20 | 3/2+ | ≈ 6500 keV | 1350E+1 20 | 100 | [E1] |  |
| 13N | 15064.6 4 | 3/2- | 0.93 keV 3 | 12693.0 4 | ≤12 | [E1] |  |
| 13N | 15064.6 4 | 3/2- | 0.93 keV 3 | 15055.2 4 | 100 6 | [M1+E2] | 0.115 22 |
| 13N | 1530E+1 20 | (3/2+) | 350 keV 150 | 1530E+1 20 | 100 | [E1] |  |

| Nucleus | Elevel(keV) | Jπ | T1/2 | Eγ(keV) | Iγ | γ mult. | γ mix. ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15N | 10066.0 2 | 3/2+ | 0.069 fs 4 | 10062.4 2 | 100.0 7 | [E1] |  |
| 15N | 10449.7 3 | 5/2- | < 0.5 keV | 10445.8 3 | <22 |  |  |
| 15N | 10533.3 5 | 5/2+ |  | 10529.3 5 | <0.3 |  |  |
| 15N | 10701.9 3 | 3/2- | 0.2 keV | 10697.8 3 | 100.0 16 | [M1+E2] | -0.180 +2-6 |

| Nucleus | Elevel(keV) | Jπ | T1/2 | Eγ(keV) | Iγ | γ mult. | γ mix. ratio | γ conv. coeff. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16O | 12049 2 | 0+ | 1.5 keV 5 | 12044.1 20 |  | [E0] |  |  |
| 16O | 12440 2 | 1- | 91 keV 6 | 12434.8 20 | 100 | [E1] |  |  |
| 16O | 12530 1 | 2- | 0.111 keV 10 | 12524.7 10 | 12.2 12 | [M2] |  |  |
| 16O | 12968.6 4 | 2- | 1.34 keV 4 | 12963.0 4 | 4.2 8 | [M2] |  |  |

| Nucleus | Elevel(keV) | Jπ | T1/2 | Eγ(keV) | Iγ | γ mult. | γ mix. ratio | γ conv. coeff. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 17O | 11078.7 9 | 1/2- | 2.4 keV 3 | 10204.6 9 | 100 | [E1] |  |  |

| Nucleus | Elevel(keV) | Jπ | T1/2 | Eγ(keV) | Iγ | γ mult. | γ mix. ratio | γ conv. coeff. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 17F | 11192.9 23 | 1/2- | 0.18 keV 3 | 10694.0 23 | 100 | [E1] |  |  |

| Nucleus | Elevel(keV) | Jπ | T1/2 | Eγ(keV) | Iγ | γ mult. | γ mix. ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20NE | 11262.3 19 | 1+ |  | 11258.9 19 | 84 5 | [M1] |  |
| 20NE | 13484 2 | 1+ | 6.4 keV 3 | 11846.5 20 | 95 |  |  |

| Nucleus | Elevel(keV) | Jπ | T1/2 | Eγ(keV) | Iγ | γ mult. | γ mix. ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 23NA | 10507.82 21 | 1/2+ | 560 eV | 10505.2 7 | 38 |  |  |
| 23NA | 10549.1 3 | 5/2+ | 540 eV | 10546.6 9 | 100 |  |  |
| 23NA | 10616.9 3 | 3/2+,5/2+ | 425 eV | 10614.3 8 | 6.1 |  |  |

| Nucleus | Elevel(keV) | Jπ | T1/2 | Eγ(keV) | Iγ | γ mult. |
| --- | --- | --- | --- | --- | --- | --- |
| 24MG | 13086.8 7 | 2+,3- | 9 keV 3 | 13083.0 7 | 0.7 3 |  |
| 24MG | 13343.8 8 | 3+ | 1.2 keV 2 | 13339.8 8 | 0.8 3 |  |

4.2.4 Testing of new detectors of type scintillator of LaBr3 at IRASM - a Co-60 Irradiation facility of IFIN-HH

The possibility to test the performance of a new scintillator detector for gamma spectrometry of LaBr3 at high rates at IRASM facility of IFIN-HH Magurele was considered. IRASM facility of IFIN-HH has a new Co-60 source of a big activity of 300 000 Ci. In this way the scintillator detector of LaBr3 could be arranged in a metallic container of dimensions of ~45 x 45 x 88 cm and could be irradiated at a maximum dose rate of 30 kGy/h. For the detectors situated at a distance of 2 m, the dose rate is 500 Gy/h.

4.3. Collaboration with Istituto di Fisica del Plasma "Piero Caldirola", CNR,  Milano, Italy

Subject:

- testing of new detectors made of new material: a detector of germanium for high resolution and two scintillator detectors of high Z material of high efficency to demonstrate the peroformance limits at high rates with real conditions

References

- [1]   NNDC,  Brookhaven National Laboratory, http://www.nndc.bnl.gov/

[2]   R. Firestone, Table of Isotopes, 8th edition, 1996